FLICKTTY : 80 COLUMNS VIC SCREEN DRIVER FOR LUNIX

-------------------------------------------------

Usage: sh0.exe ! flicktty [-0..7]

Using 80 columns with a C64 has many difficulties. Since we have the

VIC resolution of 320x200, we are limited to 4 pixels wide characters

to be shown in the 80 columns text screen. This difficulty still exists

with the 'flicktty' but not the memory and speed disadvantages, which

conventional 80-columns drivers has.

Current implementation of 'flicktty' for lunix has the throughput of

5250 characters per second. This speed is practical characteristic speed.

It is measured and calculated concerning the bad raster lines and smooth

scroll (one line up per one scroll). Using a rather optimized tricks can

make it possible to go up to 8000 cps theoretically. This would make the

connections at 57600bps flawless.

VIC memory requirement of 'flicktty' is only 3328 bytes. Two kilobytes

are used to hold partial screen data, and one kilobytes hold the font

definitions. One page of memory is for the sprites. Only one sprite is

used for the moment, which is the cursor. But the driver routine eats

seven pages of system memory, I was so lazy that I didn't deallocate the

initialization routines, which are take approximately two pages.

The idea behind the 'flicktty' is simple: Illusion. A raster interrupt

routine switches and shifts the text screen memories at each screen

refresh. This make two half screens to complement each other by using a

feature of our brains and CRTs. If your CRT has slow phosphorus and if

you have chose low contrast and intense color with proper color patterns.

Your brain cannot sense the flickker. Here the colors make important

contributions to this illusion. Our eyes have mainly two groups of sensors.

One group senses only the brightness and brain use these sensors for

shape detection. Other group gives the color information, and they are

not as fast as the brightness sensors, especially if you are lack of

vitamin A.

Technique of 'flicktty' is simple either. For the LUnix implementation

I used $a300-$afff area. With defining the first screen as the one at the

$a400-$a7e7 and the second screen at $ac00-$afe7. I initalized the screens

by using these instructions:

lda$dd00

and#$fe

sta$dd00 ; selection of $8000-$bfff for VIC display

lda#$10

sta$d011 ; 24 rows

lda#$9a

sta$d018 ; screen 1

lda#$c8

sta$d016 ; no shift

And at each raster interrupt I used these instructions to swap the screens:

lda$d018

eor#$20

sta$d018 ; display other screen

lda$d016

eor#$04

sta$d016 ; shift 4 pixels right or left

Now, together with the font, the idea is capable of showing 80 columns

wide screen. The font consists of 128 character definitions. Characters

are defined only in the most significant nybles (left four bits of a

byte).

To write something on the screen we interleave the text properly. For

example, printing 'hello world' has this representation:

odd numbered characters go to the first screen:

$a400 : 08 0c 0f 17 12 04 20 20 : hllwrd

and even numbered ones go to the second screen:

$ac00 : 05 0c 20 0f 0c 20 20 20 : el ol

Then at each 1/50 seconds one of the screens is displayed with proper

shifting:

------------------

1/50 : Ih l l w r d     I

I                I

I                I

------------------

------------------

2/50 : I e l   o l      I

I                I

I                I

------------------

.

.....and so on,

Of course this swapping cause a flicker on the screen which is somehow

annoying. But it can be reduced by several ways:

1. Mount a quality screen filter to your monitor.

2. Keep the contrast adjust of your monitor as low as possible.

3. Keep the color intensity as high as possible.

4. Adjust the brightness inbetween low and middle.

5. Choose proper colors (you have only 8 choices with 'flicktty')

6. Wear sunglasses (in that case increase the contrast adjust)

7. Take no more vitamin A ;-)

As for the usage and implementation of 'flicktty', I can say that it

is simple and limited. It's simple because it only directs its input

to the screen. For example, these usages are possible:

sh0.exe ! flicktty

cat <unix-text-file> ! unix2cbm ! flicktty

ed ! flicktty

The implementation is limited because the screen driver mimics only the

tty device. That is, no movement up or down, no color changes, and no more

reverse characters.

But it has some special character codes to refresh the color ram.

For instance pressing control-1 refreshs it.

Upon exit (which is the situation of no input) it recovers the old

screen.

Current implementation for LUnix has also other drawbacks. For example

there are screen flushes when there is a access to disk-drive. Also if

an application has long atomic processes, it also cause flushes. The

scroller within the 'flicktty' has also a long atomic area. which reduces

the multitasking a little bit. But this is not a big problem because it

can scroll 95 lines in a second. And finally it gives you only 24 rows

of text. Also note that when you scroll long texts you cannot follow it.

This is caused from the idea, not the code. You simple see the unscrolled

part for the 1/50th of a second.

Most of these drawbacks can be avoided for later releases of LUnix.

They are caused from the resource conflicts, therefore they are easy to

be corrected. But as for the last one, simply there is no solution for it.

Actually there is one similar to FLI routine, but eats 63% of CPU time.

For the color patterns you only have 8 different selections. You can

select one of them like this:

sh0.exe ! flicktty -3

instead of 3 you can enter a number 0 to 7, default is 0.

the numbers has these color patterns:

number  font color      background color

------- --------------- ----------------

0    yellow          light-blue

1    light-green     light-blue

2    yellow          purple

3    light-green     purple

4    yellow          green

5    light-blue      blue

6    green           blue

7    cyan            blue

Note that border color same as the background color. Furthermore, the font

colors are not the same as I give above, they are mixed colors caused by

the flickering.

In conclusion it can be said that the use of 'flicktty' is the only

viable solution to 80 columns problem, for limited memory and slow C64

machines.

Ilker Ficicilar

filker@newton.physics.metu.edu.tr

e068214@orca.cc.metu.edu.tr

http://newton.physics.metu.edu.tr/~filker/cbm.html

---