Internals doc for CC65

Stacks:

-------

The program stack used by programs compiled with CC65 is located in high

memory.  The stack starts there and grows down.  Arguments to functions, local

data etc are allocated on this stack, and deallocated when functions exit.

The program code and data is located in low memory. The heap is located

between the program code and the stack. The default size for the parameter

stack is 2K, you may change this by declaring an externally visible variable

named named _stksize that holds the new stack size:

unsigned _stksize = 4*1024;		/* Use 4K stack */

Note: The size of the stack is only needed if you use the heap, or if you

call the stack checking routine (_stkcheck) from somewhere in your program.

When calling other functions, the return address goes on the normal 6502

stack, *not* on the parameter stack.

Registers:

----------

Since CC65 is a member of the Small-C family of compilers, it uses the notion

of a 'primary register'.  In the CC65 implementation, I used the AX register

pair as the primary register.  Just about everything interesting that the

library code does is done by somehow getting a value into AX, and then calling

some routine or other.  In places where Small-C would use a secondary

register, top-of-stack is used, so for instance two argument function like

integer-multiply work by loading AX, pushing it on the stack, loading the

second value, and calling the internal function.  The stack is popped, and the

result comes back in AX.

Calling sequences:

------------------

C functions are called by pushing their args on the stack, and JSR'ing to the

entry point.  (See ex 1, below) If the function returns a value, it comes back

in AX.  NOTE!!!  A potentially significant difference between the CC65

environment and other C environments is that the CALLEE pops arguments, not

the CALLER.  (This is done so as to generate more compact code) In normal use,

this doesn't cause any problems, as the normal function entry/exit conventions

take care of popping the right number of things off the stack, but you may

have to worry about it when doing things like writing hand-coded assembly

language routines that take variable numbers of arguments.  More about that

later.

Ex 1:  Function call:  Assuming 'i' declared int and 'c' declared

char, the following C code

i = baz(i, c);

in absence of a prototype generates this assembler code.  I've added

the comments.

lda	_i 		; get 'i', low byte

ldx	_i+1		; get 'i', hi byte

jsr	pushax		; push it

lda	_c 		; get 'c'

ldx	#0 		; fill hi byte with 0

jsr	pushax		; push it

ldy	#4     	       	; arg size

jsr	_baz		; call the function

sta	_i  		; store the result

stx	_i+1

In presence of a prototype, the picture changes slightly, since the

compiler is able to do some optimizations:

lda	_i  	      	; get 'i', low byte

ldx	_i+1	      	; get 'i', hi byte

jsr	pushax	      	; push it

lda	_c  	      	; get 'c'

jsr    	pusha 	      	; push it

jsr	_baz	      	; call the function

sta	_i  	      	; store the result

stx	_i+1

Note that the two words of arguments to baz were popped before it exitted.

The way baz could tell how much to pop was by the argument count in Y at call

time.  Thus, even if baz had been called with 3 args instead of the 2 it was

expecting, that would not cause stack corruption.

There's another tricky part about all this, though.  Note that the args to baz

are pushed in FORWARD order, ie the order they appear in the C statement.

That means that if you call a function with a different number of args than it

was expecting, they wont end up in the right places, ie if you call baz, as

above, with 3 args, it'll operate on the LAST two, not the first two.

Symbols:

--------

CC65 does the usual trick of prepending an underbar ('_') to symbol names when

compiling them into assembler.  Therefore if you have a C function named

'bar', CC65 will define and refer to it as '_bar'.

Systems:

--------

Supported systems at this time are: C64, C128, Plus/4, CBM 600/700, the newer

PET machines (not 2001), and the Apple ][ (thanks to Kevin Ruland, who did the

port).

C64:  	The program runs in a memory configuration, where only the kernal ROM

is enabled. The text screen is expected at the usual place ($400), so

54K of memory are available to the program.

C128: 	The startup code will reprogram the MMU, so that only the kernal ROM

is enabled. This means, there are 41K of memory available to the

program.

Plus/4:	Unfortunately, the Plus/4 is not able to disable only part of it's

ROM, it's an all or nothing approach. So, on the Plus/4, the program

has only 28K available (16K machines are detected and the amount of

free memory is reduced to 12K).

CBM 600/700:

The C program runs in a separate segment and has almost full 64K of

memory available.

PET:  	The startup code will adjust the upper memory limit to the installed

memory. However, only linear memory is used, this limits the top to

$8000, so on a 8032 or similar machine, 31K of memory are available to

the program.

APPLE2:	The program starts at $800, and of RAM is $8E00, so 33.5K of memory

(including stack) are available.

Note: The above numbers do not mean that the remaining memory is unusable.

However, it is not linear memory and must be accessed by other, nonportable

methods. I'm thinking about a library extension that allows access to the

additional memory as a far heap, but these routines do not exist until now.

Inline Assembly:

----------------

CC65 allows inline assembly by a special keyword named "asm". Inline assembly

looks like a function call. The string in parenthesis is output in the

assembler file.

Example, insert a break instruction into the code:

asm ("\t.byte\t$00")

Note: The \t in the string is replaced by the tab character, as in all other

strings.

Pseudo variables:

-----------------

There are two special variables available named __AX__ and __EAX__. These

variables must never be declared (this gives an error), but may be used as any

other variable. However, accessing these variables will access the primary

register that is used by the compiler to evaluate expressions, return

functions results and pass parameters.

This feature is useful with inline assembly and macros. For example, a macro

that reads a CRTC register may be written like this:

#define wr(idx) (__AX__=(idx),asm("\tsta\t$2000\n\tlda\t$2000\n\tldx\t#$00"),__AX__)

An obvious problem here is that macro definitions may not use more than one

line.